| TYRE TIPS – Chapter 4 Installation Guidelines |  |
| --- | --- |
| 1. | Please check to make sure all tyres purchased has arrived and is undamaged. Verify that all merchandise is the correct size and type you ordered. Be sure to open and check all packages with regard to sealed wheels in boxes. Also make sure all needed hardware (caps, lug nuts, bolts, centering rings, etc.) has been included with your order unless otherwise stated. Refer back to the original invoice if you are unsure. |
| 2. | If you purchased directional Tyres or asymmetric Tyres, the first step before installing them is to match each Tyre and wheel to its final position on your vehicle. For directional Tyres, refer to the rotation arrow branded on the Tyre's sidewall - there will be two with right side rotation and two with left side rotation. The arrow indicates the direction in which the Tyre should turn. For asymmetric Tyres, all Tyres should show sidewall branding indicating "Side Facing Outward / or Showing OUTSIDE" and can be mounted on either side of the vehicle. |
| 3. | NOTE: if you only purchased wheels, test fitting for clearance must be done prior to mounting the Tyres. First, slightly loosen the lug nuts with an appropriate lug nut wheel spanner. Next, jack the vehicle up until the Tyre clears the floor. Support the vehicle with jack stands, making sure that the stands rest on a flat surface. Carefully remove lugs and check the condition of the vehicle's lug studs or wheel bolts as you remove them. If you feel any resistance or see any roughness after removing the wheels, correct it before reinstalling the wheels. Remove any temporary retaining devices, like stud clips, used to hold brake rotors and drums in place before the wheels were installed at the fitment center. They may interfere with the flush fitment of your wheels against the brake hubs. If aftermarket wheels have previously been used on the vehicle, verify that the previous wheel's hub centering rings have been removed from the hubs. If need be you might have to add Spacers in order for the wheel to clear the brakes calipers. |
| 4. | After removing your old wheels, it may be necessary to clean the hub area with a wire brush to remove any loose rust or dirt. This will ensure a clean and even mounting surface and that the wheel will sit completely flat against the drum. It is recommended that you retain your original lug hardware and store it with the spare wheel. Original equipment lug hardware often differs from aftermarket hardware supplied when fitting after market wheels, and may be required when mounting the spare Tyre on the vehicle. |
| 5. | Check the fit of the wheel onto the hub of the vehicle. When installing each wheel, make sure the wheel fits flush against the hub area and that there is no gap between the hub and the mounting surface of the wheel. If the wheel will not go on all the way, DO NOT FORCE IT ON! Check and see if anything is obstructing the wheel. If there is any doubt, please see professional assistance or contact one of our sales-agents. Hub centering rings (if required for your vehicle) should already be installed in the center of the wheels. Sometimes, it is necessary to use tape to retain the rings in the wheels during shipping. Please remove any tape before attempting to mount the wheel on the vehicle. If the rings fell out of the wheel during shipment, re-install them with beveled side facing outward. Some rings are designed to be an interference fit, and may require a rubber mallet to install. The bolt circle of the wheel must match that of your vehicle and the wheel must make full contact to the flat surface of the hub. If the wheel does not match up to the bolt circle of the vehicle, or the wheel does not have full contact to the mounting surface, Please contact the agents where you have purchased it from for further assistance. |
| 6. | For the next inspection, it will be necessary to install the wheel by snugging down the lug nuts or bolts in order to verify disc brake caliper clearance. Start threading the lugs with your fingers and tighten them until they are "finger tight." If there is any resistance, STOP! DO NOT FORCE THE LUG / BOLT ON! Inspect the lug stud and nut (or hub and bolt) to see if the threads are clean or obstructed. If the lug nut or bolt appears obstructed or does not match the thread pitch of your hubs, try another one. If another lug doesn't thread any better, something is wrong! Contact a fitment center & they will verify that you have the correct hardware for your wheel application. Also, do not coat the lug hardware with grease or oil (especially the seat--the portion of the lug hardware that actually contacts the wheel). Only after the lugs have been installed by hand until "finger tight" should you snug them down with your Wheel Spanner. Spin the assembly to check that it clears all brake and suspension parts. Then lower the vehicle until the Tyre touches the pavement for final tightening. |
| 7. | IMPORTANT! Wheel Lug Nut Torque. Proper installation requires that the wheel lug / Bolt & Nut torque be set to the recommended specification for your vehicle. These torque specifications can be found in your vehicle's manual or obtained from your vehicle dealer. The torque table below may be used only as a general guide. Finish tightening the lugs down with an accurate torque wrench. Use a crisscross sequence (see figure below) until they have reached their proper torque value, then finish lowering the vehicle. |
| Wheel Lug Tightening / Loosening Patterns |  |
| Hardware Size; Torque. 10MM; 45-55 Ft Lbs. 12MM; 80-100 Ft Lbs. 14MM; 110-140 Ft Lbs. 7/16"; 70-80 Ft Lbs. 1/2"; 80-100 Ft Lbs. 9/16"; 135-145 Ft Lbs.; Size of Bolt or Stud; Wrench Turns. 14 x 1.5mm; 7.5. 12 x 1.5mm; 6.5. 12 x 1.25mm; 8. 1/2"; 8. 7/16"; 8.. |
| NOTE: Improper lug torque can result in stripped lug nuts, broken wheel studs, a wheel loosening, or can cause the wheel, brake rotor or brake drum to distort. If you are unsure, have the lug hardware torque-checked by a qualified installation center. Recheck the lug torque within the first 100 Kilometers or 2-3 days following installation. NOTE: Since the thickness of an alloy wheel can differ from original equipment wheels, also verify that the lug nuts or bolts will engage the threads. Refer to the table above to determine the number of turns or the depth of engagement typical for your stud or bolt size. |  |
| 8. | After installing all four wheels & Tyres, lower the vehicle & push down gently on the vehicle at all four corners (using the Fender). This will help to settle the suspension. Secure vehicle with the emergency brake & turn the front wheels side to side (lock to lock) to check for any obstructions. Have an assistant verify that no part of the fender or fenderwell makes contact with the Tyre or wheel, If it does you might have to roll your fenders. DO NOT DRIVE WITH THE FENDERS TOUCHING THE TYRES! This will cut into your tyres & cause a blowout! It can also damage the vehicles body. Check all four wheels for proper clearance. Before driving down the road, drive in & out of your driveway in both directions to double-check for clearance one final time. If everything appears to fit, drive off slowly & listen for any unusual sounds. If any are heard, stop the vehicle immediately until the cause is determined. These steps will ensure that you don't have any problems due to improper fitment. It will also make exchanging an incorrect fitment much faster & easier & will save you from potential damage to your vehicle or the wheels & Tyres. |
| 9. | If you purchased a mounted wheel & Tyre package, your Tyres will not be inflated to your vehicle's specifications - due to weather, temperature & shipping variables. You will need to adjust the inflation pressures to meet the needs of your specific vehicle. Refer to the door placard or owner's manual for minimum recommended settings by axle. You may choose to adjust the inflation to meet your specific ride, handling, or Tyre wear requirements. It is recommended that these adjustments be made in increments of only 2-3 PSI. One of our sales-agents can also provide you with a good starting point for inflation based on your personal needs. |

I think its very important that we all have our rims & tyres fitted correctly prior to us driving off. If they are not correctly fitted they could cause fatal accidents.  So from myself & The team of NAZ KAR 13 PERFORMANCE WHEEL & TYRES. Should you have any queries please do not hesitate to email us on nazkar13@gmail.com . Until our next issue, Keep your wheels tightened well.

Yours in Tyre Safety & Supply.

Nazeer

NAZ KAR 13 Performance Wheel & Tyre CC

(CC – 2007/065003/23)

Wholesale Tyres Direct to Public.

Cell – 084 523 5949 / 076 962 9786

Fax – 086 564 3194

Email : nazkar13@gmail.com

The Direct, Cheaper & Safer Tyre Supplier!